Instructions

(instructions Video on our YouTube channel)

#1 The surface needs to be clean, dry and free of dust.

#2 Tape any area you don’t want glue.

#3 Apply an even layer of glue with the help of a small toothed spatula and remove any excess glue.

#4 Sprinkle rubber granules on top of the glue, and gently even out the surface with the aid of a spatula

#5 Press the granules down firmly after the glue is applied ( a piece of wood is ideal)  and then press again at least once more 1-2 hours later to stop the granules from being raised by the expanding glue.

Repeating this step several times will give a more even finish.

#6 In warm, dry conditions, the glue requires 7 days to dry and harden before being used.  During colder, damper conditions the glue can take considerably longer before drying.

#7 When the glue is completely dry, any excess rubber granules can be brushed off.